1)In order to turn ON this Click board™, the user has to press the onboard push-button labeled as

PWRKEY down for a period longer at least 500ms, which represents the Ignition (Power-On) button.

2) It supports automatic baud rate detection, operates at 115200 bps by default configuration,

and is used for data transmission and exchanging AT commands with the host MCU.

USIM Interface : Support 1.8V USIM card

3)pin definitions

BC66-NA is equipped with a total of 58 pins, including 44 LCC pins and 14 LGA pins

Power Supply Pins BC66-NA provides two VBAT pins for connection with an external power supply. The table below describes the module's VBAT and ground pins.

| PIN NO | Name |
| --- | --- |
| 1, 27, 34, 36, 37, 40, 41 | GND |
| 7 | PWRKEY |
| 15 | RESET (ACTIVE LOW) |
| 18 | RXD |
| 17 | TXD |
| 42 | VBAT_BB (power supply for module baseband part) |
| 43 | VBAT_RF (power supply for module RF part) |

AT+CTZU=1   ---->   Enable automatic time zone update

AT+CCLK=1   ---->   Get current time

AT+QCCLK   ---->  set and get current date time and UTC

AT+CGSN=1 ----> for requesting imei no

AT+CSQ   -------> signal strength

List of TCP/IP command:

List of TCP/IP Related AT Commands

AT Command      Description

[1] AT+QIOPEN       Open a Socket Service      *

[2] AT+QICLOSE      Close a Socket Service     *

[3] AT+QISTATE      Query Socket Service Status

[4] AT+QISEND       Send Hex/Text String Data  *

[5] AT+QIRD         Retrieve the Received TCP/IP Data

[6] AT+QISENDEX     Send Hex String Data

[7] AT+QISWTMD      Switch Data Access Modes

[8] AT+QPING        Ping a Remote Server

[9] AT+QNTP         Synchronize Local Time through NTP Server

[10] AT+QIDNSGIP    Get IP Address by Domain Name

[11] AT+QIDNSCFG    Configure DNS Server Address

[12] AT+QICFG               Configure Optional Parameters

[13] AT+QIGETERROR   Query the Last Error Code

List of SSL Related AT Commands

[1] AT+QSSLCFG       Configure Parameters of an SSL Context

[2] AT+QSSLOPEN      Open an SSL Socket to Connect a Remote Server

[3] AT+QSSLSEND      Send Data through SSL Connection

[4] AT+QSSLCLOSE     Close an SSL Connection